TOWN OF BARNSTABLE

A D D E N D U M # 1

DATE:                         April 5, 2017

TO:                               All Bidders

FROM:                        Johanna Boucher, Chief Procurement Officer

PROJECT	2017 Barnstable Dog Park Project, IFB dated 4/3/17

BID DUE DATE:         APRIL 28, 2017, 2 PM

PRE-BID MEETING:  4/21/17, 2 PM, Project site, entrance of Hathaway Pond, Phinney’s Lane, Barnstable, MA

The following items shall modify or be added to the Bid Documents and Project Manual.  This addendum forms part of the Contract Documents and modifies the original bidding documents.  Portions of the Bidding and Contract Documents not altered by this addendum remain in full force.  These addendum form part of the Contract Documents and as such, ALL BIDDERS SHOULD ACKNOWLEDGE IT ON THE FORM FOR GENERAL BID.  FAILURE TO DO SO COULD RESULT IN THE REJECTION OF YOUR BID.

GENERAL QUESTIONS:

- Page one of the bid document states that the bid is due 4/13/17, but the legal ad states that it is due 4/28/17, 2pm

Answer:   The title page of the bid has a typo.  The bid is due 4/28/17, no later than 2 pm to the Chief Procurmeent Officer, 230 South Street, 3rd Floor, Hyannis, MA

Johanna Boucher, Chief Procurement Officer

Johanna.boucher@town.barnstable.ma.us, 508-862-4741

SECTION 09770 - SPECIAL WALL SURFACING - ALTERNATE 1 WORK

PAR T  1 :  G ENER AL

1.01 Summary

A. Section Includes: Durable, decorative wall panels with smooth or textured finishes. Mounting hardware, adhesives, accessories and trims.

B. Related Sections:

1. Division 06 Section: Rough and Finish Carpentry.

2. Division 09 Section: Gypsum Board Assemblies.

1.02 References

A. General: Standards listed by reference, including revisions by issuing authority, form a part of this specification section to the extent indicated.

Standards listed are identified by issuing authority, authority abbreviation, designation number, title or other designation established by issuing authority. Standards subsequently referenced herein are referred to by issuing authority abbreviation and standard designation.

B. ASTM International:

1. ASTM E84 Standard Test Method for Surface Burning Characteristics of Building Materials.

2.  ASTM D256 Standard Test Methods for Determining the Izod Pendulum Impact Resistance of Plastics.

3.  ASTM D638 Standard Test Method for Tensile Properties of Plastics.

4.  ASTM D790 Standard Test Methods for Flexural Properties of Unreinforced and Reinforced Plastics and Electrical

Insulating Materials.

5.  ASTM D2583 Standard Test Method for Indentation Hardness of Rigid Plastics by Means of a Barcol Impressor.

1.03 System Description

A. Performance Requirements: Provide durable, decorative wall panels which have been manufactured and installed to maintain performance criteria stated by manufacturer without defects, damage or failure.

1.04 Submittals

A. Product Data: Submit manufacturer’s product data, storage, handling and preparation requirements and installation instructions.

B. Shop Drawings: Submit shop drawings showing layout, profiles and product components, including anchorage, accessories,

finish colors, patterns and textures. Indicate location and dimension of joints and fastener attachment.

C. Samples: Submit selection and verification samples for finishes, colors and textures. Submit 2 samples of each type of panel, trim and fastener.

General: Firm experienced in successful production of wall system similar to that indicated for the Project. Quality Assurance/Control Submittals: Submit the following:

1. Test Reports: Certified test reports showing compliance with specified performance characteristics and physical properties.

2. Certificates:

a. Submit manufacturer’s certificate that products meet or exceed specified requirements.

b. Submit certificate of installer’s qualifications.

3. Manufacturer’s Instructions: Manufacturer’s installation instructions. F. Closeout Submittals: Submit the following:

1. Operations and Maintenance Data: Operation and maintenance data for installed products in accordance with Division 01 Closeout Submittals (Maintenance Data and Operation Data) Section. Include methods for maintaining installed products and precautions against cleaning materials and methods detrimental to finishes and performance.

2. Warranty documents specified herein.

1.05 Quality Assurance

A. Installer Qualifications: Installer shall have a minimum of 5 years experience with composite wall panel work similar in scope

and size to this project.

B. Pre-installation Meetings: Host on site pre-installation meeting.  Attendees to include installer, general contractor, and owner.  Review procedures for wall surfacing installation

1.06 Delivery, Storage & Handling

A. General: Comply with Division 01 Product Requirements Section.

B. Lead Time: Comply with manufacturer’s ordering instructions and lead time requirements to avoid construction delays.

C. Delivery: Deliver materials in manufacturer’s original, unopened, undamaged containers with identification labels intact.

D. Storage and Protection: Store materials protected from exposure to harmful environmental conditions and at temperature and humidity conditions recommended by the manufacturer.

1. Store panels indoors.

2. Lay panels flat. Do not stand panels on edge.

3. Protect panels from moisture.

4. Do not store panels in contact with the floor or against an outside wall.

5. Do not remove protective film from panel surface until after installation (if applicable).

6. Maintain optimum storage conditions of 60-75 degrees F (16-24 degrees C) at 35 - 55% relative humidity. Avoid extremes in temperature and humidity.

E. Handling: Remove foreign matter from face of panel by using a soft bristle brush, avoiding abrasive action.

1.07 Project/Site Conditions

A. Environmental Requirements:

1. Installation shall not begin until building is enclosed, permanent heating and cooling equipment is in operation and residual moisture from plaster, concrete or terrazzo work has dissipated.

2. Install panels between 60 degrees F - 75 degrees F (15 - 24 degrees C) and relative humidity below 55%, ideally at the same conditions as the room’s normal operating temperatures after building is occupied.

3. Provide ventilation to disperse fumes during application of adhesive as recommended by adhesive manufacturer.

4. Do not install wall system until normal lighting conditions exist. Normal lighting conditions are described as those in place when the project is finished.

B. Field Measurements: Verify actual measurements/openings by field measurements before fabrication; show recorded measurements on shop drawings. Coordinate field measurements and fabrication schedule with construction progress to avoid construction delays.

1. Wall, ceilings, floors and openings must be level, plumb, straight, in-line and square

1.08 Warranty

A. Project Warranty: Refer to Conditions of the Contract for project warranty provisions.

B. Manufacturer’s Warranty: Submit, for Owner’s acceptance, manufacturer’s standard warranty document executed by authorized company official. Manufacturer’s warranty is in addition to, and not a limitation of, other rights Owner may have under Contract Documents.

PAR T 2 : P RO DUCT S

2.01 Composite Wall Panels

A. Manufacturer: Nudo or approved

B. Proprietary Products/Systems: Composite wall & ceiling panels, including the following:

FibeLite® FRP Wall Panels:

FiberLite: Fiberglass Reinforced Plastic (FRP) panel

Texture: Smooth, High Pressure Laminate (HPL): NA Thickness: 0.090"

Fire-Rating Class: A

Color: To be selected by the Owner from Manufacturers standard colors

Moldings To be selected by the Owner from Manufacturers standard

PAR T 3 : EXEC U T I O N

3.03 Preparation

A. Comply with the instructions and recommendations of the durable, decorative wall panel manufacturer.

Site Verification of Conditions: Verify substrate conditions, which have been previously installed under other sections, are acceptable for product installation in accordance with manufacturer’s instructions. Verify that site conditions are acceptable for installation of durable, decorative wall panels. Examine back-up surfaces to determine that corners are plumb and straight, surfaces are smooth, uniform, clean and free from foreign matter, nails are countersunk and joints and cracks are filled flush and smooth with the adjoining surface. Do not proceed with installation of durable, decorative wall panels until unacceptable conditions are corrected.

3.04 Installation:  Install in strict accordance to Manufacturers recommendations

END OF SECTION 09770

Copyright 2005 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 07311 - ASPHALT SHINGLES

- GENERAL

      - SECTION REQUIREMENTS

        - Submittals:  Product Data and Samples.

        - Identify each bundle of shingles with appropriate markings of UL or another testing and inspecting agency acceptable to authorities having jurisdiction.

          - Exterior Fire-Test Exposure:  ASTM E 108 or UL 790, Class A.

          - Wind-Resistance-Test Characteristics:  ASTM D 3161 or UL 997, passed.

        - Warranties:  Provide standard manufacturer's written warranty, signed by manufacturer agreeing to promptly repair or replace asphalt shingles that fail in materials or workmanship within 30 years from date of Substantial Completion.

- PRODUCTS

      - ASPHALT SHINGLES

        - Fiberglass Shingles:  ASTM D 3462 and as follows:

          - Laminated-Strip Asphalt Shingles:  Laminated, multi-ply overlay construction, mineral-granule surfaced, and self-sealing.  Straight cut butt edge.

          - Shingles to be CertainTeed Landmark Atlantic Blue Shingle:  This is the Hyannis Water Departments standard shingle product.

          - Shingles shall be installed as required to meet high wind prone areas.  Install with a minimum 6 nails pattern as shown on manufactures instructions for high wind prone areas.

      - ACCESSORIES

        - Starter Strip Shingles:  Manufacturers standard starter strip shingles required

        - Felts:  Two layers 15# asphalt-saturated organic felts.

        - Self-Adhering Sheet Underlayment:  ASTM D 1970, SBS-modified asphalt; mineral-granule or slip-resisting-polyethylene surfaced; with release paper backing; cold applied.

        - Ridge Vent:  Rigid UV-stabilized plastic ridge vent as recommended by the roofing manufacturer; for use under ridge shingles.

        - Asphalt Roofing Cement:  ASTM D 4586, Type II, asbestos free.

        - Roofing Nails:  Stainless-steel Shingle nails, as recommended by the roofing manufacturer, of sufficient length to penetrate at least 1/8 inch through plywood sheathing.

        - Sheet Metal Flashing and Trim:  Comply with requirements in Division 7 Section "Sheet Metal Flashing and Trim."

          - Sheet Metal:  Aluminum

          - Fabricate sheet metal flashing and trim to comply with recommendations in SMACNA's "Architectural Sheet Metal Manual."

          - Drip Edge:  Formed sheet metal with at least a 2-inch roof deck flange and a 1-1/2-inch fascia flange with a 3/8-inch drip at lower edge.  Color White

- EXECUTION

      - INSTALLATION

        - Comply with recommendations in ARMA's "Residential Asphalt Roofing Manual" and with asphalt shingle recommendations in NRCA's "The NRCA Roofing and Waterproofing Manual."

        - Apply self-adhering sheet underlayment at eaves and rakes from edges of roof to at least 36 inches inside exterior wall line.

        - Apply self-adhering sheet underlayment at valleys extending 18 inches on each side.

        - Install valleys complying with ARMA and NRCA instructions.  Construct woven and closed-cut valleys.

        - Install metal flashings and other sheet metal to comply with requirements in Division 7 Section "Sheet Metal Flashing and Trim," recommendations in ARMA's "Residential Asphalt Roofing Manual," and asphalt shingle recommendations in NRCA's "The NRCA Roofing and Waterproofing Manual."

        - Install first and remaining courses of asphalt shingles stair-stepping diagonally across roof deck with manufacturer's recommended offset pattern at succeeding courses, maintaining uniform exposure.

END OF SECTION 07311

End of Addenda No. 1